Table S1 HR classification of toxic or adverse effects of chemicals.

| Hazard rating | Description of effects/Hazard category | Example of chemicals |
| --- | --- | --- |
| 1 | - No known adverse health effect - ACGIH A5 carcinogens - Not classified as toxic or harmful | Sodium chloride, butane, butyl acetate, calcium carbonate |
| 2 | - Reversible effects to the skin, eyes or mucous membranes, not sever enough to cause serious health impairment - ACGIH A4 carcinogens - Skin sensitizer and skin irritants | Acetone, butane, acetic acid (10% concentration), barium salts, aluminum dust |
| 3 | - Possible human or animal carcinogens or mutagens, but for which data is inadequate - ACGIH A3 carcinogens - IARC Group 2B - Corrosive (pH 3 to 5 or 9 to 11), respiratory sensitizers, harmful chemicals | Toluene, xylene, ammonia, butanol, acetaldehyde, acetic anhydride, aniline, antimony |
| 4 | - Probable human carcinogens or mutagens based on animal studies - ACGIH A2 carcinogens - NTP Group B - IARC Group 2A - Very corrosive (pH 0 to 2 or 11.5 to 14) - Toxic chemicals | Formaldehyde, cadmium, methylene chloride, thylene oxide, acrylonitrile, 1,3-butadiene |
| 5 | - Known human carcinogens, mutagens or teratogens - ACGIH A1 carcinogens - NTP Group A - IARC Group 1 - Very toxic chemicals | Benzene, benzaidine, lead, arsenic, beryllium, bromine, vinyl chloride, mercury, crystalline silica |

Table S2 Exposure Rating

| E/OEL | Exposure Rating (ER) |
| --- | --- |
| <0.1 | 1 |
| 0.1 to <0.5 | 2 |
| 0.5 to <1.0 | 3 |
| 1.0 to <2.0 | 4 |
| ≥ 2.0 | 5 |

E = weekly exposure; OEL = Occupational Exposure Limit

Table S3 Classificationof Risk Rating*

| Risk Rating | Ranking |
| --- | --- |
| 1 | Negligible |
| 2 | Low |
| 3 | Medium |
| 4 | High |
| 5 | Very high |

*